Servo Exercise

Add a brief paragraph describing the function of a servo.  What is in it, how does it work, what commands does it take (pulse width), what feedback does it give.  Maybe borrow a few figures showing the internal components?

# Lab Exercise

This exercise is intended to make sure that everyone has some experience using the HandyBoard to control a servo.  If you have programmed the HB for servo operation already, then these steps will be simple to complete.

## Prepare the Experiment

- Locate the c-code to run the servo on the lab computers

C:\IC\libs\RCservo

- Save a copy of servoExample.c that you can use to make changes to the code.

- Unload any previous files on the HB and load the three files

servoExample.c	(a copied version)

servo_a5.icb

servo_a7.icb

- Put the white (signal) lead into the digital 9 port (the top slot)

- Put the red and black leads into the +5 and gnd buses respectively

## Test Limit Conditions

The file servoExample.c provides a pulse of a specified width (in clock ticks) to Digital Port 9 proportional to the knob position on the HB. In your robot, the knob value will be replaced with a value based upon the sensor readings.  In this exercise, you will explore the pulse width values that can be given to your servo.

- The default min and max values are 650 and 4700 clock ticks.  Modify the program to determine if this matches the range of the servo that you are testing.

- Test several intermediate positions to determine linearity of the relationship between pulse width and servo angle.

- Show your friendly TA how you found the limits and use the knob to prove that your software limits correspond to the hardware limits.

# Questions

What are two benefits of using a SERVO in your robot

What are two limitations of using a SERVO in your robot.

Compare a servo motor to a DC motor and a Stepper Motor.