Servo Exercise

This exercise is intended to make sure that everyone has some experience using the HandyBoard to control a servo.  If you have programmed the HB for servo operation already, then these steps will be simple to complete.

- Locate the c-code to run the servo on the lab computers

C:\IC\libs\RCservo

- Save a copy of servoExample.c that you can use to make changes to the code.

- Unload any previous files on the HB and load the three files

servoExample.c	(a copied version)

servo_a5.icb

servo_a7.icb

- Put the white (signal) lead into the digital 9 port (the top slot)

- Put the red and black leads into the +5 and gnd buses respectively

The file servoExample.c gives digital 9 and timer-out 3 a value based on the position of the knob.  When used for your skiing robot, the knob value will be replaced with a value based on the sensor readings.  In this exercise, you will determine the min and max values that can be given to your servo.

- The default min and max values are 650 and 4700.  Use your imagination and creativity to determine if a different range is needed.

- Show your friendly TA how you found the limits and use the knob to prove that your software limits correspond to the hardware limits.